Ladders & Stepladders

Safety

-  Read and follow the manufacturer's instructions label affixed to the ladder if you are unsure how to use the ladder.

-  Do not use ladders that have loose rungs, cracked or split side rails, missing rubber foot pads, or are otherwise visibly damaged.

-  Keep ladder rungs clean and free of grease. Remove buildup of material such as dirt or mud.

-  Do not place ladders in a passageway or doorway without posting warning signs or cones that direct pedestrian traffic away from the ladder. Lock the doorway that you are blocking with the ladder and post signs that will detour traffic away from your work.

-  Do not place a ladder at a blind corner or doorway without diverting foot traffic by blocking or roping off the area.

-  Allow only one person on the ladder at a time.

-  Face the ladder when climbing up or down it.

-  Maintain a three-point contact by keeping both hands and one foot or both feet and one hand on the ladder at all times when climbing up or down the ladder.

-  When performing work from a ladder, face the ladder and do not lean backward or sideways from the ladder.

-  Do not jump from ladders or step stools.

-  Do not stand on tables, chairs, boxes or other improvised climbing devices to reach high places, use the ladder or stepstool.

-  Do not stand on the top two rungs of any ladder.

-  Do not stand on a ladder that wobbles, or that leans to the left or right of center.

-  When using a straight or extension ladder, extend the top of the ladder at least 3 feet above the edge of the landing.

-  Secure the ladder in place by having another Employee hold it if it cannot be tied to the structure.

-  Do not move a rolling ladder while someone is on it.

-  Do not place ladders on barrels, boxes, loose bricks, pails, concrete blocks or other unstable bases.

-  When operating at heights above 6 feet on platforms such as scaffolding, aerial lifts, and scissor lifts all Employees must receive fall protection training, this includes the use of a harness and lanyard.

- Climb down a ladder one rung at a time.

- Do not climb with tools in your hand.  Use a tool belt.

Ladders & Stepladders

Safety

-  Read and follow the manufacturer's instructions label affixed to the ladder if you are unsure how to use the ladder.

-  Do not use ladders that have loose rungs, cracked or split side rails, missing rubber foot pads, or are otherwise visibly damaged.

-  Keep ladder rungs clean and free of grease. Remove buildup of material such as dirt or mud.

-  Do not place ladders in a passageway or doorway without posting warning signs or cones that direct pedestrian traffic away from the ladder. Lock the doorway that you are blocking with the ladder and post signs that will detour traffic away from your work.

-  Do not place a ladder at a blind corner or doorway without diverting foot traffic by blocking or roping off the area.

-  Allow only one person on the ladder at a time.

-  Face the ladder when climbing up or down it.

-  Maintain a three-point contact by keeping both hands and one foot or both feet and one hand on the ladder at all times when climbing up or down the ladder.

-  When performing work from a ladder, face the ladder and do not lean backward or sideways from the ladder.

-  Do not jump from ladders or step stools.

-  Do not stand on tables, chairs, boxes or other improvised climbing devices to reach high places, use the ladder or stepstool.

-  Do not stand on the top two rungs of any ladder.

-  Do not stand on a ladder that wobbles, or that leans to the left or right of center.

-  When using a straight or extension ladder, extend the top of the ladder at least 3 feet above the edge of the landing.

-  Secure the ladder in place by having another Employee hold it if it cannot be tied to the structure.

-  Do not move a rolling ladder while someone is on it.

-  Do not place ladders on barrels, boxes, loose bricks, pails, concrete blocks or other unstable bases.

-  When operating at heights above 6 feet on platforms such as scaffolding, aerial lifts, and scissor lifts all Employees must receive fall protection training, this includes the use of a harness and lanyard.

- Climb down a ladder one rung at a time.

- Do not climb with tools in your hand.  Use a tool belt.

Ladders & Stepladders

Safety

-  Read and follow the manufacturer's instructions label affixed to the ladder if you are unsure how to use the ladder.

-  Do not use ladders that have loose rungs, cracked or split side rails, missing rubber foot pads, or are otherwise visibly damaged.

-  Keep ladder rungs clean and free of grease. Remove buildup of material such as dirt or mud.

-  Do not place ladders in a passageway or doorway without posting warning signs or cones that direct pedestrian traffic away from the ladder. Lock the doorway that you are blocking with the ladder and post signs that will detour traffic away from your work.

-  Do not place a ladder at a blind corner or doorway without diverting foot traffic by blocking or roping off the area.

-  Allow only one person on the ladder at a time.

-  Face the ladder when climbing up or down it.

-  Maintain a three-point contact by keeping both hands and one foot or both feet and one hand on the ladder at all times when climbing up or down the ladder.

-  When performing work from a ladder, face the ladder and do not lean backward or sideways from the ladder.

-  Do not jump from ladders or step stools.

-  Do not stand on tables, chairs, boxes or other improvised climbing devices to reach high places, use the ladder or stepstool.

-  Do not stand on the top two rungs of any ladder.

-  Do not stand on a ladder that wobbles, or that leans to the left or right of center.

-  When using a straight or extension ladder, extend the top of the ladder at least 3 feet above the edge of the landing.

-  Secure the ladder in place by having another Employee hold it if it cannot be tied to the structure.

-  Do not move a rolling ladder while someone is on it.

-  Do not place ladders on barrels, boxes, loose bricks, pails, concrete blocks or other unstable bases.

-  When operating at heights above 6 feet on platforms such as scaffolding, aerial lifts, and scissor lifts all Employees must receive fall protection training, this includes the use of a harness and lanyard.

- Climb down a ladder one rung at a time.

- Do not climb with tools in your hand.  Use a tool belt.

SAFETY QUIZ

Ladders

- What are some signs that a ladder should not be used? (circle all that apply)

- It has steps missing.

- It only has three legs.

- The steps are covered with grease.

- It is sturdy and you have a coworker to hold the ladder.

- When climbing a ladder what kind of contact should you maintain?

  - Hands free is okay, as long as you have two feet on it.

  - One hand and one foot at all times.

  - Three points of contact two hands and one foot or vice versa.

  - You must keep both hands and both feet on the ladder at all times.

- Circle all the items that are suitable for use to reach high places, besides a ladder or a step stool.

a.  Tables.

b.  Boxes.

c.  Chairs.

d.  A coworkers shoulders.

- Which of the following are unsafe practices when using a ladder? (circle all that apply)

a.  Jumping from ladders.

b.  Standing on the top rung of a ladder.

c.  Having multiple people on the ladder at one time.

d.  Having a coworker hold the ladder while you climb up always maintaining 3 points of contact.

e.  Climbing up the ladder backwards.

- When operating at heights above 6 feet what precautions should be taken?

a. 	Employees should receive fall protection training.

b.  	Employees should use a harness and lanyard.

c.  	Employees should be equipped with a parachute.

d.      No precautions need to be taken it is perfectly safe.

Print Name  _____________________________________

Sign Name  _____________________________________

SAFETY QUIZ

Ladders

- What are some signs that a ladder should not be used? (circle all that apply)

a. 	It has steps missing.

b. 	It only has three legs.

- The steps are covered with grease.

- It is sturdy and you have a coworker to hold the ladder.

- When climbing a ladder what kind of contact should you maintain?

  - Hands free is okay, as long as you have two feet on it.

  - One hand and one foot at all times.

  - Three points of contact two hands and one foot or vice versa.

  - You must keep both hands and both feet on the ladder at all times.

- Circle all the items that are suitable for use to reach high places, besides a ladder or a step stool.

a.  Tables.

b.  Boxes.

c.   Chairs.

d.  A coworkers shoulders.

- Which of the following are unsafe practices when using a ladder? (circle all that apply)

a.  Jumping from ladders.

b.  Standing on the top rung of a ladder.

c.  Having multiple people on the ladder at one

time.

d.  Having a coworker hold the ladder while you           climb up always maintaining 3 points of contact.

e.  Climbing up the ladder backwards.

- When operating at heights above 6 feet what precautions should be taken?

a. 	Employees should receive fall protection training.

b.  	Employees should use a harness and lanyard.

c.  	Employees should be equipped with a parachute.

d.     	No precautions need to be taken it is perfectly safe.

Print Name  _____________________________________

Sign Name  _____________________________________

SAFETY QUIZ

Ladders

- What are some signs that a ladder should not be used? (circle all that apply)

- It has steps missing.

- It only has three legs.

- The steps are covered with grease.

- It is sturdy and you have a coworker to hold the ladder.

- When climbing a ladder what kind of contact should you maintain?

  - Hands free is okay, as long as you have two feet on it.

  - One hand and one foot at all times.

  - Three points of contact two hands and one foot or vice versa.

  - You must keep both hands and both feet on the ladder at all times.

- Circle all the items that are suitable for use to reach high places, besides a ladder or a step stool.

a.  	Tables.

b.  	Boxes.

c.   	Chairs.

d.  	A coworkers shoulders.

- Which of the following are unsafe practices when using a ladder? (circle all that apply)

a.  Jumping from ladders.

b.  Standing on the top rung of a ladder.

c.  Having multiple people on the ladder at one    time.

d.  Having a coworker hold the ladder while you climb up always maintaining 3 points of contact.

e.    Climbing up the ladder backwards.

- When operating at heights above 6 feet what precautions should be taken?

a. 	Employees should receive fall protection training.

b.  	Employees should use a harness and lanyard.

c.  	Employees should be equipped with a parachute.

d.    	 No precautions need to be taken it is perfectly safe.

Print Name  _____________________________________

Sign Name  _____________________________________